CONFIGURE IPV6 ADDRESSING

Tasks

1. Open your Windows or Linux system, use the command line to assign a static IPv6 address to your primary network adapter, and verify the configuration using ipconfig (Windows) or ifconfig/ip (Linux).

### Assigning a Static IPv6 Address & Verification

- Steps Performed:

  - Opened Command Prompt as an Administrator.

  - Executed the command to assign a static IPv6 address (2001:db8::10/64) to the active network adapter (Wi-Fi 2):

Command:

netsh interface ipv6 add address "Wi-Fi 2" 2001:db8::10/64

  - Verified the configuration by running the ipconfig command.

- Verification Result:

  - Successfully confirmed that the static IPv6 address (2001:db8::10) was correctly bound and active on the primary wireless network adapter (Wi-Fi 2).

2. Configure a unique local IPv6 address (starting with 'fd') on your system and confirm connectivity by pinging another device on your LAN that also has a unique local IPv6 address. Hint: Use the ping6 or ping -6 command for IPv6 testing.

### Unique Local IPv6 Address (ULA) & Ping Test

- Steps Performed:

  - Configured a Unique Local IPv6 Address (ULA) starting with 'fd' (fd00::1) on the active network adapter (Wi-Fi 2) using the command line:

netsh interface ipv6 add address "Wi-Fi 2" fd00::1/64

  - Verified the ULA configuration via ipconfig, confirming that fd00::1 was successfully bound to Wi-Fi 2.

  - Tested connectivity to another device on the local area network using the IPv6 ping command:

Command:

ping -6 fd00::2

- Observation / Result:

  - The command executed successfully, though it returned a "Destination host unreachable" response (100% packet loss) because no second device with the specified ULA address (fd00::2) was active on the local network at the time of testing.

3. Set up a temporary IPv6 address on your system and check if it changes after a reboot or network disconnect/reconnect. Document the steps and the observed address behavior in a text file.

### Temporary IPv6 Address Setup & Behavior Documentation

- Objective: Set up a temporary IPv6 address (privacy extension) on the system and document its behavior after a network disconnect/reconnect or reboot.

- Steps Performed:

  - Opened the Command Prompt with Administrator privileges.

  - Executed the command to enable randomized interface identifiers (Privacy Extensions) globally:

Command:

netsh interface ipv6 set global randomizeidentifiers=enabled

  - Checked the network adapter configuration using ipconfig to verify the application of the setting.

- Observed Address Behavior (Documentation):

  - Privacy Mechanism: Temporary IPv6 addresses utilize randomized interface identifiers instead of static MAC-based EUI-64 addresses to prevent tracking across different networks.

  - Behavior After Network Disconnect/Reconnect or Reboot: Upon disconnecting and reconnecting from the network, or after a system reboot, temporary IPv6 addresses automatically expire and regenerate a new randomized address, whereas stable static or global/ULA addresses remain persistent.

4. Simulate a Zomato-style search: Write a small script (any language) that stores an array of restaurant names and their IPv6 addresses, then allows the user to input a partial name and returns the matching restaurant's IPv6 address.

### Zomato-Style Restaurant IPv6 Search Script

- Objective: Write a small script that stores an array/dictionary of restaurant names with their IPv6 addresses, accepts a partial name input from the user, and returns the matching restaurant's IPv6 address.

- Python Script Implementation:

def zomato_ipv6_search():

# Storing restaurant names and their corresponding IPv6 addresses

restaurants = {

"Pizza Hut": "2001:db8:1::10",

"Domino's Pizza": "2001:db8:1::11",

"Burger King": "2001:db8:1::12",

"Subway": "2001:db8:1::13",

"McDonald's": "2001:db8:1::14",

}

print("--- Zomato IPv6 Restaurant Directory ---")

query = input("Enter partial restaurant name to search: ").lower()

found = False

print("\nSearch Results:")

for name, ipv6 in restaurants.items():

if query in name.lower():

print(f"Restaurant: {name} | IPv6 Address: {ipv6}")

found = True

if not found:

print("No matching restaurants found.")

if __name__ == "__main__":

zomato_ipv6_search()

**How It Works:**

* Uses a Python dictionary (`restaurants`) to map restaurant names to their respective IPv6 addresses.

* Takes user input, converts it to lowercase for case-insensitive matching, and checks if the partial query exists within any restaurant name.

* Displays the matching restaurant name and its IPv6 address.

5. Use ChatGPT or Copilot to generate a sample IPv6 subnetting scenario (with at least 3 subnets), and then manually assign a valid IPv6 address from one of those subnets to your system. Paste the AI-generated scenario and your configuration steps in a text file.

### IPv6 Subnetting Scenario & Manual System Configuration

- Task Requirement: Generate a sample IPv6 subnetting scenario with at least 3 subnets and manually assign a valid IPv6 address from one of the subnets to the system adapter.

- AI-Generated Scenario (ChatGPT / Copilot Style):

Scenario: An organization receives an IPv6 global unicast address block with a prefix of 2001:db8:abc0::/48. The network administrator needs to subdivide this block into standard /64 subnets to allocate distinct networks for three primary department offices:

  - Engineering Department Subnet: 2001:db8:abc0:1::/64

  - Sales Department Subnet: 2001:db8:abc0:2::/64

  - Administration Department Subnet: 2001:db8:abc0:3::/64

- Manual Configuration Steps:

  - Opened Command Prompt with Administrator privileges.

  - Selected a valid IPv6 address from the Engineering Department Subnet (2001:db8:abc0:1::100/64) and assigned it to the active network adapter (Wi-Fi 2) using the command:

Command:

netsh interface ipv6 add address "Wi-Fi 2" 2001:db8:abc0:1::100/64

  - Verified the successful manual binding of the subnet address by running ipconfig.