ETHERNET STANDARD AND SPECIFICATION

Tasks

1. Draw a simple diagram showing the pin layout of an RJ-45 connector and label the color codes for the T568A and T568B Ethernet wiring standards.

### RJ-45 Connector Pin Layout & Wiring Standards (T568A vs T568B)

- RJ-45 Connector Pin Layout

- Color Codes Comparison Table:

| Pin Number | T568A Standard Color | T568B Standard Color |
| --- | --- | --- |
| Pin 1 | White/Green (Striped Green) | White/Orange (Striped Orange) |
| Pin 2 | Solid Green | Solid Orange |
| Pin 3 | White/Orange (Striped Orange) | White/Green (Striped Green) |
| Pin 4 | Solid Blue | Solid Blue |
| Pin 5 | White/Blue (Striped Blue) | White/Blue (Striped Blue) |
| Pin 6 | Solid Orange | Solid Green |
| Pin 7 | White/Brown (Striped Brown) | White/Brown (Striped Brown) |
| Pin 8 | Solid Brown | Solid Brown |

2. Create a table comparing the key specifications of Ethernet standards 10BASE-T, 100BASE-TX, and 1000BASE-T, including speed, cable type, maximum cable length, and connector type.

## Comparison of Ethernet Standards

| Specification | 10BASE-T | 100BASE-TX | 1000BASE-T |
| --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1000 Mbps (1 Gbps) |
| Cable Type | Cat3 or higher UTP | Cat5 UTP | Cat5e or higher UTP |
| Maximum Cable Length | 100 metres | 100 metres | 100 metres |
| Connector Type | RJ-45 | RJ-45 | RJ-45 |
| Pairs Used | 2 pairs | 2 pairs | 4 pairs |

### Short Explanation

- 10BASE-T: Provides a speed of 10 Mbps and is used in older Ethernet networks.

- 100BASE-TX: Provides a speed of 100 Mbps and is commonly known as Fast Ethernet.

- 1000BASE-T: Provides a speed of 1 Gbps and is known as Gigabit Ethernet.

3. Given a scenario where you need to connect two laptops directly using Ethernet, explain which cable (straight-through or crossover) you would use and why. Think about how transmit and receive pairs are wired in each cable type.

### Connecting Two Laptops Directly via Ethernet

- Recommended Cable Type: Crossover Cable.

- Explanation & Reasoning (Based on Transmit and Receive Pairs):

  - Device Classification (DTE): Laptops and computers are classified as DTE (Data Terminal Equipment) devices. This means their internal network interface card (NIC) pins are wired identically—they use Pins 1 & 2 to transmit data (TX) and Pins 3 & 6 to receive data (RX).

  - The Problem with Straight-Through Cables: If you use a standard straight-through cable, both laptops will try to send data on pins 1 & 2 and listen on pins 3 & 6 at the same time. Because the transmit and receive lines don't cross, the signals crash or go nowhere, resulting in no connection.

  - Why Crossover Works: A crossover cable internally crosses the wire pairs at one end (connecting Pin 1 to Pin 3, and Pin 2 to Pin 6). This ensures that the Transmit (TX) pins of Laptop A connect directly to the Receive (RX) pins of Laptop B, and vice versa, enabling seamless two-way communication.

(Note: While modern laptops feature Auto-MDI/MDIX technology which can automatically detect and adjust for straight-through cables, the traditional and academically correct answer for a direct DTE-to-DTE connection is a crossover cable).

4. List three real-world devices in your home or college that use Ethernet connections, and for each, specify the likely Ethernet standard (e.g., 100BASE-TX, 1000BASE-T) they support.

### Real-World Devices & Their Supported Ethernet Standards

- 1. Home Wi-Fi Router / Network Switch

  - Supported Standard: 1000BASE-T (Gigabit Ethernet)

  - Why: Modern broadband routers use gigabit ports to handle heavy high-speed internet traffic coming from the ISP and distribute fast wired connections to local devices without creating a network bottleneck.

- 2. College Lab Desktop PC / Workstation

  - Supported Standard: 1000BASE-T (Gigabit Ethernet)

  - Why: Motherboards in modern desktop computers come equipped with built-in Gigabit NICs (Network Interface Cards) to allow fast file sharing, access to heavy server databases, and smooth internet usage on campus.

- 3. Smart TV / Network Printer

  - Supported Standard: 100BASE-TX (Fast Ethernet)

  - Why: Streaming high-definition videos or sending print commands does not require ultra-high gigabit bandwidth, so these devices typically use 100 Mbps ports, which are more than enough to handle their daily data requirements efficiently.