IP ADDRESSING & SUBNETTING

Tasks

1. List the IP address classes (A, B, C, D, E) and give one real-world example of a device or service that might use each class. Write your answers in a table format.

### IP Address Classes & Real-World Examples

| IP Class | Leading Bits / First Octet Range | Default Subnet Mask | Real-World Example of Device or Service |
| --- | --- | --- | --- |
| Class A | 1 to 127 (0xxxxxxx) | 255.0.0.0 | Large global enterprise or ISP core networks (e.g., Apple's internal networks). |
| Class B | 128 to 191 (10xxxxxx) | 255.255.0.0 | Medium-to-large university campus or corporate networks. |
| Class C | 192 to 223 (110xxxxx) | 255.255.255.0 | Standard home Wi-Fi routers and small office/home office (SOHO) local networks. |
| Class D | 224 to 239 (1110xxxx) | N/A (Multicast) | IP Multicast streaming services (e.g., live IPTV broadcasting or video conference multicasting). |
| Class E | 240 to 255 (1111xxxx) | N/A (Experimental) | Reserved for experimental, research, or developmental purposes (not used in production). |

2. Given the IP address 192.168.10.45 with subnet mask 255.255.255.0, calculate the network address, broadcast address, and the range of valid host addresses. Show your working steps for each calculation.

### Subnet Calculations for 192.168.10.45

- Given Information:

  - IP Address: 192.168.10.45

  - Subnet Mask: 255.255.255.0 (which represents a /24 prefix)

### Calculations & Working Steps:

#### 1. Network Address

- Working Step:

  - The network address is found by performing a bitwise AND operation between the IP address and the subnet mask.

  - Since the subnet mask is 255.255.255.0, it allows the first three octets (192.168.10) to remain untouched and sets the final host octet to 0.

- Result: 192.168.10.0

#### 2. Broadcast Address

- Working Step:

  - In a standard /24 subnet, the broadcast address uses the network's base prefix, and all bits in the host portion (the final octet) are set to binary 1 (which equals 255 in decimal).

- Result: 192.168.10.255

#### 3. Range of Valid Host Addresses

- Working Step:

  - Usable host IPs always start right after the network address and end right before the broadcast address.

  - Start IP: Network address + 1 (192.168.10.1)

  - End IP: Broadcast address - 1 (192.168.10.254)

- Result: 192.168.10.1 to 192.168.10.254

3. Create a subnetting plan for a small food delivery startup like Zomato that needs 4 separate subnets: one for Delivery Partners, one for Restaurants, one for Customers, and one for Admins. Assign a unique subnet (with network address and subnet mask) for each group using the 10.0.0.0/24 network.

### Subnetting Plan for Food Delivery Startup (10.0.0.0/24)

- Requirement: 4 separate subnets needed (Delivery Partners, Restaurants, Customers, Admins).

- Calculation Summary:

  - To create 4 subnets, we borrow 2 bits from the host portion (22=4).

  - New Subnet Mask: /26 or 255.255.255.192

  - Block Size / Increment: 64 addresses per subnet.

- Subnet Allocation Table:

| Subnet Group | Network Address | Subnet Mask | Usable Host IP Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1. Delivery Partners | 10.0.0.0 | 255.255.255.192 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| 2. Restaurants | 10.0.0.64 | 255.255.255.192 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| 3. Customers | 10.0.0.128 | 255.255.255.192 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| 4. Admins | 10.0.0.192 | 255.255.255.192 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

4. You are given the following scenario: A college WiFi network uses the IP range 172.16.0.0/16. The admin wants to create at least 20 subnets for different buildings. Calculate the new subnet mask and the number of hosts available per subnet. Constraint: Show your calculation steps and final answers.

### College Wi-Fi Network Subnetting (172.16.0.0/16)

- Given Information:

  - Base Network: 172.16.0.0/16 (Class B network, prefix length = 16 bits)

  - Requirement: At least 20 subnets for different buildings.

### Calculation Steps:

#### 1. Determine the Number of Bits to Borrow (n)

- To find how many bits need to be borrowed from the host portion to create at least 20 subnets, we use the formula:

2^n ≥ Required Subnets

- Testing values for n:

  - If n = 4: 2^4 = 16 subnets (This is less than 20, so it is insufficient).

  - If n = 5: 2^5 = 32 subnets (This is greater than or equal to 20, which satisfies the requirement).

- Therefore, we must borrow 5 bits (n = 5).

#### 2. Calculate the New Subnet Mask

- Original network prefix: /16

- New network prefix: 16 + 5 = /21

- Converting the /21 prefix into a decimal subnet mask:

  - First octet: 255

  - Second octet: 255

  - Third octet: 5 bits set to 1, followed by 3 zeros (11111000 in binary) →128 + 64 + 32 + 16 + 8 =248

  - Fourth octet: 0

- New Subnet Mask: 255.255.248.0

#### 3. Calculate the Number of Hosts Available per Subnet

- Total bits in an IPv4 address = 32 bits.

- New network prefix = 21 bits.

- Remaining bits for hosts (h) = 32 - 21 = 11 bits.

- Total IP addresses per subnet = 2^11 = 2,048.

- Usable host addresses (excluding the network address and broadcast address) =2,048 - 2 = 2,046 hosts.

### Final Answers:

- New Subnet Mask: 255.255.248.0 (or /21)

- Number of Subnets Created: 32 subnets

- Number of Hosts Available per Subnet: 2,046 usable hosts

5. A friend sends you a message asking why their device with IP 192.168.1.300 is not connecting to the network. Write a short explanation identifying the error and what a valid IP address in that subnet should look like.

### Troubleshooting IP Address Error (192.168.1.300)

- Identification of the Error:

  - The IP address 192.168.1.300 is invalid, which is why the device cannot connect to the network.

  - Reason: In an IPv4 address, each of the four decimal octets must fall strictly between 0 and 255. Since 300 exceeds the maximum possible value of 255, the operating system and router reject it as an impossible address.

- What a Valid IP Address Should Look Like:

  - To remain in the same subnet (192.168.1.x), the final octet needs to be corrected to any available number between 1 and 254.

  - Example of a valid IP address: 192.168.1.50 (or any value from 192.168.1.1 to 192.168.1.254).