IP ROUTING AND ROUTING PROTOCOLS

Tasks

1. Draw a simple network diagram showing three devices (like your phone, a friend's laptop, and a smart TV) connected through a router, and label the IP addresses for each device and the router's interfaces.

Network Diagram Representation:

🌐 ROUTER

┌──────────────────┐

│               LAN IP:                      │

│           192.168.1.1                     │

└───────┬──────────┘

│

┌───────────┼───────────┐

│           │           │

▼           ▼           ▼

📱 Phone    💻 Laptop    📺 Smart TV

IP:         IP:           IP:

192.168.1.2 192.168.1.3   192.168.1.4

IP Address Allocation Breakdown:

- Router (Default Gateway Interface): 192.168.1.1 (Subnet Mask: 255.255.255.0)

- Device 1 (Phone): 192.168.1.2

- Device 2 (Friend's Laptop): 192.168.1.3

- Device 3 (Smart TV): 192.168.1.4

2. Simulate manual IP routing by writing a step-by-step explanation of how a packet would travel from your laptop to a friend's computer in a different subnet, including which router(s) it passes through and how the routing decision is made. Use made-up IP addresses and show the routing table entries that would be checked at each hop.

## Manual IP Routing Simulation

### Network Diagram

💻 My Laptop

IP: 192.168.1.10

|

| 192.168.1.1

▼

🔵 Router 1

G0/0: 192.168.1.1

G0/1: 10.0.0.1

|

| 10.0.0.0/30

▼

🔵 Router 2

G0/0: 10.0.0.2

G0/1: 192.168.2.1

|

|

▼

💻 Friend's Computer

IP: 192.168.2.10

### Step-by-step packet travel

1. Laptop sends the packet
My laptop has IP 192.168.1.10 and wants to send data to my friend's computer 192.168.2.10.

2. Laptop checks the subnet
My laptop is on 192.168.1.0/24, while my friend's computer is on 192.168.2.0/24.
Because they are in different subnets, the laptop sends the packet to its default gateway: 192.168.1.1.

3. Router 1 checks its routing table

| Destination Network | Next Hop | Interface |
| --- | --- | --- |
| 192.168.1.0/24 | Directly Connected | G0/0 |
| 10.0.0.0/30 | Directly Connected | G0/1 |
| 192.168.2.0/24 | 10.0.0.2 | G0/1 |

Router 1 finds 192.168.2.0/24 and forwards the packet to Router 2 (10.0.0.2).

4. Router 2 checks its routing table

| Destination Network | Next Hop | Interface |
| --- | --- | --- |
| 10.0.0.0/30 | Directly Connected | G0/0 |
| 192.168.2.0/24 | Directly Connected | G0/1 |

Router 2 sees that 192.168.2.10 belongs to its directly connected network 192.168.2.0/24.

5. Packet reaches the friend's computer
Router 2 forwards the packet through G0/1 to 192.168.2.10.

### Routing decision

The router checks the destination IP address and looks for the best matching route in its routing table. It then forwards the packet through the appropriate interface or next-hop router.

Flow:
192.168.1.10 → Router 1 → Router 2 → 192.168.2.10 📦➡️

3. Create a routing table for a fictional Zomato-style delivery network with three branches in different cities, each on a different subnet. Specify the network address, subnet mask, next hop, and interface for each route.

## Routing Table – Fictional Zomato-Style Delivery Network

Suppose a food-delivery company has three branches in different cities. Each branch uses a different subnet.

### Network Setup

🔵 Main Router

/      |      \

/       |       \

▼        ▼        ▼

Jaipur     Delhi     Mumbai

Branch     Branch     Branch

192.168.10.0  .20.0      .30.0

### Routing Table

Branch / City; Network Address; Subnet Mask; Next Hop; Interface. Jaipur; 192.168.10.0; 255.255.255.0; 10.0.0.2; G0/0. Delhi; 192.168.20.0; 255.255.255.0; 10.0.0.6; G0/1. Mumbai; 192.168.30.0; 255.255.255.0; 10.0.0.10; G0/2.

### Explanation

- Network Address: Identifies the destination branch network.

- Subnet Mask: 255.255.255.0 means each branch has a /24 network.

- Next Hop: The IP address of the next router where the packet should be sent.

- Interface: The router port used to forward the packet.

Example:
If a packet is going to a computer in Mumbai with IP 192.168.30.10, the router checks its routing table, finds 192.168.30.0/24, and forwards the packet through G0/2 to the next hop 10.0.0.10.

This shows how a Zomato-style network can route delivery/order data between branches in different cities.

4. Compare static routing and dynamic routing protocols (like RIP or OSPF) by listing 3 pros and 3 cons of each, using examples from real-world apps or services you use (e.g., how a food delivery app might benefit from dynamic routing).

### Comparison of Static Routing vs. Dynamic Routing Protocols

- Task Requirement: Compare static routing and dynamic routing protocols (like RIP or OSPF) by listing 3 pros and 3 cons of each, using real-world examples (such as a food delivery app).

#### 1. Static Routing

Static routes are manually configured by a network administrator. They do not change unless the administrator manually updates them.

- Pros:

  - High Security & Control: Because routes are manually entered, administrators have full control over the exact path packets take, preventing unauthorized rerouting.

  - Zero Overhead: Consumes no CPU, memory, or bandwidth resources on routers for route calculations or updates.

  - Predictability: Ideal for simple, small, and stable networks where paths never change.

- Cons:

  - Not Scalable: Extremely tedious and prone to human error when managing large, growing networks with hundreds of subnets.

  - No Fault Tolerance: If a link or router fails, static routes cannot automatically reroute traffic; it requires manual intervention.

  - High Maintenance: Any network topology change requires manual reconfiguration across all routers.

- Real-World Analogy: Like a printed physical map or a delivery driver who memorizes a single fixed route to a specific restaurant and cannot adapt if a road gets closed or blocked by construction.

#### 2. Dynamic Routing Protocols (e.g., OSPF, RIP)

Dynamic routing protocols automatically discover network destinations, exchange routing information, and adapt to changes using algorithms.

- Pros:

  - Scalability: Automatically handles large and complex enterprise networks with thousands of nodes effortlessly.

  - Fault Tolerance & Redundancy: If a primary link fails, protocols like OSPF automatically recalculate and reroute traffic through an alternate healthy path within seconds.

  - Plug-and-Play Adaptability: New routers or subnets automatically announce themselves to the network without manual configuration updates.

- Cons:

  - Resource Intensive: Consumes router CPU, memory, and link bandwidth to constantly monitor health and exchange routing table updates.

  - Security Risks: Vulnerable to malicious route injection attacks or misconfigurations that can cause routing loops if proper authentication isn't used.

  - Complex Troubleshooting: Diagnosing network issues can be difficult because the path traffic takes is determined dynamically by algorithms rather than fixed rules.

- Real-World Analogy / Food Delivery App Example: Exactly like Zomato or Google Maps GPS navigation, which continuously monitors live traffic congestion, accidents, or road closures, and dynamically reroutes delivery partners through the fastest available path in real-time.

5. Use ChatGPT or another AI tool to generate a sample configuration for setting up OSPF routing between two routers, then explain in your own words what each configuration line  Paste the AI-generated config and annotate each part with a short description.

## OSPF Configuration Between Two Routers

### Network Diagram

💻 LAN 1

192.168.1.0/24

|

|

Router 1

G0/0: 192.168.1.1

G0/1: 10.0.0.1

|

| 10.0.0.0/30

|

G0/1: 10.0.0.2

Router 2

G0/0: 192.168.2.1

|

|

💻 LAN 2

192.168.2.0/24

### Router 1 Configuration

Router> enable

Router# configure terminal

Router(config)# router ospf 1

Router(config-router)# router-id 1.1.1.1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.3 area 0

Router(config-router)# end

### Router 2 Configuration

Router> enable

Router# configure terminal

Router(config)# router ospf 1

Router(config-router)# router-id 2.2.2.2

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

Router(config-router)# network 10.0.0.0 0.0.0.3 area 0

Router(config-router)# end

### Explanation of Each Line

Configuration; Meaning. enable; Enters privileged mode.. configure terminal; Opens global configuration mode.. router ospf 1; Starts OSPF process 1.. router-id 1.1.1.1; Gives Router 1 a unique OSPF ID.. router-id 2.2.2.2; Gives Router 2 a unique OSPF ID.. network 192.168.1.0 0.0.0.255 area 0; Tells Router 1 to advertise its LAN network through OSPF.. network 192.168.2.0 0.0.0.255 area 0; Tells Router 2 to advertise its LAN network through OSPF.. network 10.0.0.0 0.0.0.3 area 0; Enables OSPF on the link connecting the two routers.. area 0; Places the network in OSPF backbone area 0.. end; Exits configuration mode..

### In simple words

OSPF allows Router 1 and Router 2 to automatically learn about each other's networks. So Router 1 can learn 192.168.2.0/24 from Router 2, and Router 2 can learn 192.168.1.0/24 from Router 1.

The 0.0.0.255 and 0.0.0.3 values are wildcard masks, not normal subnet masks.