NETWORK CABLE AND CONNECTORS

Tasks

1. Draw a labeled diagram of a showing the color order of wires in the T568B standard and labeling the RJ-45 connector pins.

### Cat6 Ethernet Cable Diagram & T568B Standard

- T568B Standard Color Order (Pin 1 to 8):

  - Pin 1: White/Orange (Striped Orange)

  - Pin 2: Solid Orange

  - Pin 3: White/Green (Striped Green)

  - Pin 4: Solid Blue

  - Pin 5: White/Blue (Striped Blue)

  - Pin 6: Solid Green

  - Pin 7: White/Brown (Striped Brown)

  - Pin 8: Solid Brown

- RJ-45 Connector Pins Layout

2. List three types of network cables (such as Cat5e, Cat6, and fiber optic) and for each, mention one real-world app or device (like a WiFi router, Zomato POS terminal, or IPL streaming server) where it would be used, explaining why.

### Three Types of Network Cables & Real-World Use Cases

- 1. Cat5e Cable

  - Real-World App/Device: Home Wi-Fi Router / Standard Office PC.

  - Why it is used: It supports data transfer speeds up to 1 Gbps over short distances and is cost-effective, making it ideal for standard residential internet connections and regular office networking tasks.

- 2. Cat6 Cable

  - Real-World App/Device: Zomato POS (Point of Sale) Terminal.

  - Why it is used: It provides higher bandwidth (up to 10 Gbps) and features better internal twisting and insulation that significantly reduces crosstalk, ensuring fast, secure, and uninterrupted billing transactions.

- 3. Fiber Optic Cable

  - Real-World App/Device: IPL Live Streaming Server / ISP Data Backbone.

  - Why it is used: It transmits data using light pulses instead of electrical signals, allowing massive bandwidth capacity and high-speed data transmission over extremely long distances without any signal degradation or lag.

3. Watch a YouTube video showing how to crimp an RJ-45 connector onto a network cable, then write down the step-by-step process in your own words and mention two common mistakes to avoid.

### RJ-45 Crimping Process & Common Mistakes

- Step-by-Step Crimping Process:

  - Measure and Strip the Jacket: Cut the desired length of the Ethernet cable and use the wire stripper to remove about 3 cm of the outer plastic jacket without damaging the inner conductors.

  - Untwist and Straighten Wires: Cut away the central plastic separator (in Cat6) and untwist all four pairs of wires, then straighten them out thoroughly.

  - Arrange in T568B Order: Align the 8 wires strictly in the T568B standard color order (White/Orange, Orange, White/Green, Blue, White/Blue, Green, White/Brown, Solid Brown).

  - Trim the Wires Evenly: Measure the wires against the RJ-45 plug and snip the ends straight and flat so they fit evenly.

  - Insert and Crimp: Carefully slide the wires all the way into the RJ-45 connector until the copper ends reach the front tip and the outer jacket is inside the plastic clamp, then insert the plug into the crimping tool and squeeze firmly.

- Two Common Mistakes to Avoid:

  - Leaving wires too long or too short: If the wires are left too long, the outer jacket won't be clamped securely by the connector; if they are too short, they won't make proper contact with the copper pins.

  - Incorrect color sequence: Forgetting to double-check the exact pin color order before crimping, which leads to communication failures or faulty connections.

- Reference Video: How to Crimp Cat5 / Cat6 Network Patch Cables (RJ45 plugs)

4. Given the following scenario: You are setting up a gaming zone for an IPL screening event and need to connect multiple screens and consoles. Choose the appropriate cable and connector types for high-speed, reliable connections, and explain your choices.

### Gaming Zone & IPL Screening Event Setup

- Chosen Cable & Connector Types: Cat6 Ethernet Cables with RJ-45 Connectors.

- Explanation & Reasoning:

  - High Speed & Bandwidth: Cat6 cables support data transfer speeds up to 10 Gbps over shorter distances, which is essential for handling massive data streams during live IPL broadcasts and multi-console online gaming without lagging.

  - Low Latency & Reliability: Wired Ethernet connections offer a stable, interference-free connection with zero packet loss—unlike Wi-Fi, which can suffer from congestion or signal drops when a large crowd is present at an event.

  - Secure Physical Connection: RJ-45 connectors provide a lockable, sturdy termination into gaming consoles, smart TVs, and network switches, ensuring the physical setup remains secure throughout the event.

5. Find and share a photo (from your home, college, or online) of any network cable or connector in use (such as behind a WiFi router or at a café's POS counter), and describe what type it is and what devices it connects.Look for blue or yellow cables with clear plastic ends (RJ-45) or thin round cables for fiber optics.

- Type of Cable: Cat6 Ethernet Cable with clear plastic RJ-45 connectors.

- Connected Devices: It connects the Internet Service Provider (ISP) modem or broadband wall port to the WAN port of a Wi-Fi wireless router, allowing internet distribution throughout the area.